Hydraulic Performance Evaluation of Type-H Inlets

Thesis presented to

The faculty of Department of Civil and Environmental Engineering

University of Houston

In partial fulfillment

of the requirements for the degree

Master of Science

In

Civil Engineering

by:

Abhijeet Shreeram Bobde

December 2007

Hydraulic Performance Evaluation of Type-H Inlets

______________________

Abhijeet Bobde

Approved:





    
Chairman of Committee





    
Dr. Theodore Cleveland, Associate Professor,





    
Civil and Environmental Engineering

Committee Members:




   
_______________________________





    
Dr. Jerry Rogers, Associate Professor,





    
Civil and Environmental Engineering

                                          
______________________________





   
Dr. Keh Han Wang, Professor,





   
Civil and Environmental Engineering

_________________________          
_____________________________


Larry C. Witte, Associate Dean, 
 
Cumaraswamy Vipulanandan, 
Cullen College of Engineering           
Professor and Chairman, Civil and 







Environmental Engineering

Hydraulic Performance Evaluation of Type-H Inlets

An abstract of

The
Thesis presented to

The faculty of Department of Civil and Environmental Engineering

University of Houston

In partial fulfillment

of the requirements for the degree

Master of Science

In

Civil Engineering

by:

Abhijeet Shreeram Bobde

December 2007

Abstract

Use of grate type in highway drainage is common on TX-DOT highways, but there is a lack of a formal, tested methodology for the design of these inlets. This research, funded by TX-DOT defines a basis for the design of H-inlets.

Literature reported experimental data for 3 different inlet morphologies was analyzed to determine the measured factors that predict the efficiency. An efficient inlet is the one which can drain maximum water; there are other competing requirements for the design of a good inlet such as safety, strength and debris handling. So far, efficiency of inlets was studied as a function of the flow volume and slope of the flow channel. In the present research, importance was given on use of non-dimensional variables and geometric ratios particular to the inlet to relate the hydraulic efficiency to these variables. The goal is to develop design methods   generalized for inlets of similar morphologies independent of specific physical dimensions. The aim was to convert experimental data into non-dimensional charts to guide future experiments.

Acknowledgements

I take this opportunity to thank my advisor Dr. Theodore G. Cleveland Ph D., P.E. for his guidance, support and constant encouragement which has helped me in completing this thesis. He has always been there to help me in every possible way.
I am thankful to Texas Department of Transportation for funding the research.  I also extend my gratitude to Dr. Ken Han Wang and Dr. Jerry Rogers for their valuable time and guidance in improving the thesis. I thank the entire staff of Civil and Environmental Engineering Department and my colleagues Kunal Kumar, Thuy Luong, Siva Sunder and Divagar for their help.

I will forever be grateful to my Parents, back in India, who have been a source of inspiration to me and my sister who has always supported me morally.
Index

Abstract……………………………………………………………………………iv
Acknowledgements………………………………………………………………...v

Contents…………………………………………………………………………...vi
List of Figures……………………………………………………………………viii
List of Tables…………………………………………………………………........xi
Contents

1. Introduction ……..…………………………………………………..................   2
2. Literature Review


2.1 Background and scope of literature review………………………….…6


2.2 Inlet Capacity Studies- Typical Experimental Design…………….……8

2.3 Debris Handling……………………………………………... ….…….12

2.4 Findings of Prior Researchers……………………………….. ….…….13


2.4.1 Inlet Capacity – Grate Inlets……………………… ….……..13


2.5 Debris Handling – Grate Inlets………………………………. ….……18

2.7 Design Methods……………………..……………………….. ….……20


2.7.1 General Approach…………………………….…….. ….…...20
3. Problem Statement…………………………………………………… ….……..22
4. Approach

4.1 Methods: Non-Dimensionalization of literature reported experimental data…………………………………………………. ….……. ….………..24
4.2 Inlet Morphologies……………………………………………………..25

4.3 Identification of Non-dimensional Variables………………… ….…...26

4.3.1 Longitudinal Aspect ratio…………………………….….…..26

4.3.2 Effective Area ratio………………………………….. ….….33

4.3.3 Longitudinal Slope…………………………………………..37

4.3.4 Flow Width Ratio…………………………………………....37

4.3.5 Froude Number………………………………………..….…39

4.4 Flow Depth Calculations…………………………………….... ….…..45
4.5 The Proposed Model……………………………………….………….45
5. Results………………………………………...…………………….……….….50

5.1 Additive Models…..…………………………………….……………..50

5.2 Power Law Models..………………………….………………………..53

5.3 Testing the Models…………………………………….………………58
5.4 Standard Errors and Prediction Error Bounds……………………..….62
6. Conclusion and Future Work…………………………………………………...63

6.1 Design Oriented Experimental Setup………………………………….63
References………………………………………………………………...............67
Appendix – A Structural Details of the inlets…………………………………….69
Appendix – B Regression in R-console………………….………..……...............72
Appendix –C Data tables ………………………………………………..…….…84
Figures

Figure 1. Representative Type-H Inlet (grate top) near I-35 at Jarrell, Texas………3
Figure 2. Type-H Inlet (grate top), sloped entry. I-35 feeder near San Marcos, Texas……………………………………………………………………………….
.3
Figure 3. Representative Type-H Inlet (lid top). Not a TxDOT inlet………..………4 

Figure 4. Roadside grate inlet. I-35 feeder near San Marcos, Texas…..……..……..5

Figure 5. Typical physical modeling configuration………………………………….9

        Figure 6. Image of Curb and Grate inlet with debris residue…………………12

Figure 7. Typical results presentation (Woo and Jones, 1974)……………………..14
 Figure 8. Schematic (profile view) of tilted-bar inlet.  Each bar acts like a scoop. 

(Adapted from Woo and Jones, 1974)……………………………………………..16
Figure 9. Slanted-vane type grate inlet………………………………………….…17
Figure 10. Dimension of the inlet with respect to direction of flow………………27
Figure 11.Typical result graphs which were converted into numerical data……...30
Figure 12 Efficiency vs longitudinal aspect ratio for inlet P 1-1/8………………..31
Figure 13 Efficiency vs longitudinal aspect ratio for inlet P 1-7/8………………..32

Figure 14 Efficiency vs longitudinal aspect ratio for inlet CV 3-1/4……………...32     
Figure 15 Efficiency values for different values of effective area ratio of inlet 
P 1-1/8……………………………………………………………………………..34

Figure 16 Efficiency values for different values of effective area ratio of inlet
 P 1-7/8………………………………………………………………………….....35
Figure 17 Efficiency values for different values of effective area ratio of inlet
 CV 3-1/4………………………………………………………………………….36

Figure 18 Image of typical Result Graphs of Inlet Efficiencies Against Total 
Gutter Flow at Different Slopes from the Studied Literature (Burgi, 1978)………38
Figure 19 Width of flow and the inlet dimensions demonstrating the concept
 of flow width ratio…………..……………………………..…………………..….39
Figure 20 Plot of observed efficiency values and Froude number with a linear trend-

 line (inlet P 1-1/8)…………………………………………………………………42

Figure 21 Plot of observed efficiency values and Froude number with a linear trend-

 line (inlet P 1-7/8)………………………………………………………………….43

Figure 22 Plot of observed efficiency values and Froude number with a linear trend-

 line (inlet CV 3-1/4)……………………………………………………………….44

Figure 23 Cross Section of the Flow Channel……………………………………...46
Figure 24 Evaluating the consistency of P 1-1/8 Additive Model, observed versus
 calculated efficiencies……………………………...……………………………....51
Figure 25 Evaluating the consistency of P 1-7/8 Additive Model, observed versus 
calculated efficiencies……………………………………………………………....52
Figure 26 Evaluating the consistency of CV 3-1/4 Additive Model, observed
 versus calculated efficiencies……………….……...……………………………....54
Figure 27 Evaluating the consistency of P 1-1/8 Power Law Model, observed
versus calculated efficiencies……………………………………………...…..........55
Figure 28 Evaluating the consistency of P 1-7/8 Power Law Model, observed
 versus calculated efficiencies…..……………………………...…...……………..56

Figure 29 Evaluating the consistency of CV 3-1/4 Power Law Model, observed
 versus calculated efficiencies….…………………………………………...…......58
Figure 30 Using CV 3-1/4 Power Law Model to Calculate Efficiency of 
TB45 type Inlet………..………………………………………….…………….....60
Figure 31 Using CV 3-1/4 Additive Model to Calculate Efficiency of
 TB45 type Inlet………..…………………………………………………………61
Figure 32 The prediction error bounds associated with the CV 3-1/4 power law 
model ………………………………………………………………………….…62
Figure A1. Details of P 1-1/8 Type Inlet……………………………………........69
Figure A2. Details of P 1-7/8 Type Inlet…………...…….………………………70
Figure A3 Details of CV 3-1/4 Type Inlet………………….………………….…71
Figure B1 P 1-1/8 additive model regression in R-console………………………73
Figure B2 P 1-7/8 additive model regression in R-console………………………75
Figure B3 CV 3-1/4 additive model regression in R-console…………………….77
Figure B4 P 1-1/8 power law model regression in R-console……………………79
Figure B5 P 1-7/8 power law model regression in R-console……………………81
Figure B6 CV 3-1/4 power law model regression in R-console…………………83

Tables
Table 1. Comparative experimental designs…………………….………….………11
Table 2. Dimensions corresponding to longitudinal aspect ratios of inlets used..….29
Table 3. The Efficiency Model 1 and the parameters corresponding to the three
 types of inlet……………………………………………..……………………..…..48
Table 4. The Efficiency Model 2 and the Parameters Corresponding to the Three
 Types of inlet………………………………………………………………………48
Table 5. Parameters for P1-1/8 Additive Model……………………………………50
Table 6. Parameters for P1-7/8 Additive Model……………………………………52

Table 7. Parameters for CV 3-1/4 Additive Model…………………………………53

Table 8. Parameters for P1-1/8 Power Law Model…………………………………54

Table 9 Parameters for P1-7/8 Power Law Model…………………………….........56
Table 10 Parameters for CV 3-1/4 Power Law..……………………………………57
Table C1 Data adopted from Cassidy (1966) and use of CV 3-1/4 power law model
 to compute efficienc………………………………………………………………...84
Table C2 parameters of CV 3-1/4 power law model………………………………...85
Table C3 Data adopted from Cassidy (1966) and use of CV 3-1/4 additive model to
 compute efficiency…………………………………………………………….….....86

Table C2 parameters of CV 3-1/4 power law model……………………………...…87
PAGE  
viii

